| Instruction Type | Opcode (7-bit) | ALUOp (Category) | Funct Code | Final Operation (4'b) | Meaning |
| --- | --- | --- | --- | --- | --- |
| Load (ld) | 0000011 | 00 (Add) | Ignored | 0010 | ADD (Address Calc) |
| Store (sd) | 0100011 | 00 (Add) | Ignored | 0010 | ADD (Address Calc) |
| Branch (beq) | 1100011 | 01 (Sub) | Ignored | 0110 | SUB (Comparison) |
| R-Type (add) | 0110011 | 10 (Look at Funct) | 0000 | 0010 | ADD |
| R-Type (sub) | 0110011 | 10 (Look at Funct) | 1000 | 0110 | SUB |
| R-Type (and) | 0110011 | 10 (Look at Funct) | 0111 | 0000 | AND |
| R-Type (or) | 0110011 | 10 (Look at Funct) | 0110 | 0001 | OR |

| Bit Range | [31:25] | [24:20] | [19:15] | [14:12] | [11:7] | [6:0] |
| --- | --- | --- | --- | --- | --- | --- |
| Field Name | funct7 | rs2 | rs1 | funct3 | rd | Opcode |
| Purpose | Math Variant | Source Reg 2 | Source Reg 1 | Math Operation | Dest Reg | Inst Type |
| R-Type (add, sub) | 0000000 / 0100000 | rs2 | rs1 | ADD/OR/AND | rd | 0110011 |
| I-Type (ld) | Immediate | — | rs1 | Width | rd | 0000011 |
| S-Type (sd) | Immediate | rs2 | rs1 | Width | Immediate | 0100011 |
| B-Type (beq) | Immediate | rs2 | rs1 | BEQ/BNE | Immediate | 1100011 |